# RTL Development Phase

## Signal Generation and Feature Processing

### Goal

Build the foundation of the pain-classification system.

At the end of Week 1 the team should be able to generate synthetic physiological signals and extract useful features.

# Module 1: Synthetic Physiological Signal Generator

## Objective

Create digital representations of physiological features that can later be used for pain classification.

## Inputs

None (initial version)

## Outputs

Simulated:

- Alpha Power

- Beta Power

- Theta Power

- GSR Level

## Example Scenarios

### Scenario A

Low Pain

Alpha = 80

Beta = 20

Theta = 30

GSR = 15



### Scenario B

Medium Pain

Alpha = 50

Beta = 50

Theta = 40

GSR = 45



### Scenario C

High Pain

Alpha = 20

Beta = 80

Theta = 60

GSR = 85



## Verilog Tasks

Implement:

- Counter-based generators

- Test stimulus generation

- State switching

## Deliverables

- Verilog source

- Testbench

- Waveforms

- RTL schematic

# Module 2: Moving Average Filter

## Objective

Reduce fluctuations in incoming signals.

## Formula

Moving Average =

(X1 + X2 + X3 + X4)/4



## Example

Input:

80

78

85

81

Output:

81



## Verilog Tasks

Implement:

- Shift register

- Adder

- Divider

## Deliverables

- Verilog module

- Testbench

- Simulation waveforms

# Module 3: RMS Estimator

## Objective

Estimate signal strength.

## Formula

RMS =

√[(x1²+x2²+...+xn²)/n]



## Simplification

Initially:

Implement approximate RMS.

Avoid floating point.

Use:

- Fixed point arithmetic

- Integer operations

## Deliverables

- RTL

- Verilog

- Testbench

# Module 4: Feature Vector Generator

## Objective

Combine extracted features into a structured output.

## Inputs

Alpha

Beta

Theta

GSR



## Output

Feature Vector

Example:

[80,20,30,15]



## Deliverables

- Block diagram

- RTL

- Simulation

# Integration

Connect:

Signal Generator

↓

Moving Average

↓

RMS Estimator

↓

Feature Vector



# Demonstration

Show:

- Generated signals

- Filtered signals

- RMS output

- Final feature vector

# 2: Pain Classification Engine

## Goal

Convert physiological features into pain levels.

# Step 1: Pain State Definition

Define:

## Pain Level 0

Low Pain

Characteristics:

High Alpha

Low Beta

Low GSR



## Pain Level 1

Moderate Pain

Characteristics:

Balanced Alpha

Balanced Beta

Moderate GSR



## Pain Level 2

High Pain

Characteristics:

Low Alpha

High Beta

High GSR



# Step 2: Classification Table

Create truth table.

Example:

| Alpha | Beta | GSR | Pain |
| --- | --- | --- | --- |
| High | Low | Low | Low |
| Medium | Medium | Medium | Moderate |
| Low | High | High | High |

# Step 3: Comparator Design

## Objective

Compare incoming features against thresholds.

## Example Thresholds

Alpha > 70

Alpha High

Beta > 70

Beta High

GSR > 70

High Stress/Pain

## Deliverables

- Comparator module

- Simulation results

# Step 4: Pain Classification Logic

## Objective

Generate pain state.

## Logic Example

IF

Alpha High

AND

Beta Low

AND

GSR Low

THEN

Pain = Low



IF

Alpha Low

AND

Beta High

AND

GSR High

THEN

Pain = High



## Deliverables

- Truth table

- Decision diagram

- RTL implementation

# Step 5: FSM Design

## States

Low Pain

Moderate Pain

High Pain



## State Transitions

Allow movement between states according to feature changes.

## Deliverables

- FSM diagram

- State table

- Verilog implementation

# Integration

Connect:

Feature Vector

↓

Comparators

↓

Decision Logic

↓

Pain Classifier

↓

Pain State Output



# Demonstration

Show:

Scenario A:

Low Pain Input

Output:

Pain Level 0



Scenario B:

Moderate Pain Input

Output:

Pain Level 1



Scenario C:

High Pain Input

Output:

Pain Level 2



# Deliverables

- Signal Generator

- Moving Average Filter

- RMS Estimator

- Feature Vector Generator

- Comparator Modules

- Pain Classification Logic

- FSM Design

- Verilog Source Files

- Testbenches

- Simulation Waveforms

- RTL Schematics

# Expected Outcome 

A working FPGA simulation capable of:

Synthetic Physiological Features

↓

Feature Processing

↓

Pain Classification

↓

Pain State Output

